Paper Title

Given Name Surname 
dept. name of organization 
(of Affiliation)
name of organization 
(of Affiliation)
City, Country
email address or ORCID

Abstract— This paper addresses the global challenge of food security by introducing a comprehensive, low-cost Internet of Things (IoT)–based system that integrates hardware, cloud computing, and machine learning to promote precision farming and intelligent crop recommendations. The system is designed as a complete end-to-end framework that enables efficient, sustainable, and data-driven agricultural practices. At its foundation, the hardware architecture is built around the ESP32 microcontroller unit (MCU), chosen for its low power consumption, integrated Wi-Fi connectivity, and strong computational capabilities, making it ideal for remote, continuous monitoring in agricultural settings. The ESP32 is responsible for acquiring real-time soil data through a suite of sensors, including a physical soil moisture sensor that measures water availability and a simulated chemical sensor array that provides continuous readings of essential soil health indicators such as Nitrogen (N), Phosphorous (P), Potassium (K), and pH levels. Each data point collected is timestamped to ensure temporal accuracy and securely transmitted wirelessly to a centralized cloud platform. This cloud platform, built using the Flask Python framework, serves as the core hub for data ingestion, processing, storage, and visualization. Beyond basic data handling, the platform performs data enrichment by integrating real-time meteorological data—including temperature, humidity, and rainfall—fetched dynamically from reliable external Application Programming Interfaces (APIs). This fusion of static soil attributes with dynamic weather conditions results in a comprehensive and multidimensional dataset, forming the foundation for the system’s intelligent analytics layer. The machine learning (ML) component constitutes the system’s intelligence, where the enriched dataset, comprising seven key parameters (N, P, K, pH, Moisture, Temperature, Humidity, and Rainfall), is processed through a pre-trained Random Forest classification algorithm. The Random Forest model, selected for its high predictive accuracy, robustness against overfitting, and capability to manage diverse agricultural data, outputs the most suitable crop recommendation for the given real-time conditions. This enables site-specific, adaptive decision-making, helping farmers select the optimal crop based on current soil fertility and climatic factors. The paper provides a complete account of the system’s development, covering the bill of materials (BoM), circuit design, hardware integration, cloud-based application structure, and the machine learning pipeline. Furthermore, it includes a demonstration of the system’s real-world implementation, validating its scalability, cost-effectiveness, and performance. By combining IoT-based sensing, real-time cloud computation, and AI-driven intelligence, the proposed system represents a significant step toward sustainable agriculture, empowering farmers with actionable insights, optimizing resource utilization, and enhancing crop yield. This holistic approach to smart farming underscores the potential of IoT and machine learning technologies in addressing global food challenges while fostering environmentally responsible and economically viable agricultural practices.

Keywords: Internet of Things (IoT), Smart Agriculture, ESP32, Crop Recommendation, Machine Learning, Random Forest, Flask, Precision Farming, Sustainable Agriculture.

SYSTEM ARCHITECTURE

The proposed IoT-based Smart Agriculture and Crop Recommendation System is structured into three major layers—the Hardware/Perception Layer, the Transport/Cloud Layer, and the Application/Analysis Layer—each playing a vital role in ensuring efficient data flow, reliable communication, and intelligent decision-making.

At the Hardware/Perception Layer, the system is powered by the ESP32 microcontroller unit (MCU), selected for its integrated Wi-Fi connectivity, low power consumption, and sufficient computational strength to handle real-time sensing operations. This layer serves as the foundation of the architecture, responsible for environmental data acquisition from a network of sensors deployed in the agricultural field. The sensor suite includes a soil moisture sensor for measuring water content directly in the soil, along with a simulated chemical sensor array that provides continuous readings of essential soil nutrients such as Nitrogen (N), Phosphorous (P), and Potassium (K), as well as the soil pH level. Together, these sensors generate critical data that reflects the physical and chemical condition of the soil. The ESP32 processes this data locally, timestamps it for temporal accuracy, and formats it for transmission, forming the core input that drives the entire intelligent system.

The Transport/Cloud Layer serves as the backbone of the system, enabling seamless communication and centralized data management. Once the ESP32 collects the sensor data, it transmits it wirelessly via Wi-Fi to the cloud platform, which is built using the Flask Python framework. This layer handles the secure transmission, storage, and organization of incoming data in a structured database. To enhance the quality and contextual accuracy of the dataset, this layer also integrates external meteorological information—specifically temperature, humidity, and rainfall—retrieved from reliable online APIs. This integration combines static soil parameters with dynamic weather conditions, producing a comprehensive, multidimensional dataset essential for generating accurate crop predictions. Furthermore, the cloud infrastructure supports real-time synchronization between the IoT devices and the user interface, ensuring that data visualizations and analytics remain current and accessible.

Finally, the Application/Analysis Layer acts as the decision and visualization interface of the system. This layer comprises two main components: the user-facing website and the machine learning (ML) module. The web application, hosted on the Flask platform, provides a clean, interactive dashboard that allows farmers and agricultural experts to monitor soil health, view environmental conditions, and access crop recommendations in real time. The integrated ML model, powered by the Random Forest classification algorithm, forms the system’s intelligence core. It processes the enriched dataset consisting of seven key parameters (N, P, K, pH, Moisture, Temperature, Humidity, and Rainfall) and predicts the most suitable crop for the current soil and environmental conditions. This predictive output is displayed on the web interface in a user-friendly format, enabling informed and data-driven agricultural decisions.

In essence, the three-layer architecture ensures seamless end-to-end functionality, from sensing and transmission to intelligent analysis and visualization. The modular design allows for scalability, where additional sensors or data sources can be integrated with minimal modification. By linking real-time IoT sensing, cloud-based data enrichment, and machine learning intelligence, the proposed system provides a robust, reliable, and practical technological framework for modern precision farming.

Hardware Design

The hardware design of the proposed IoT-based Smart Agriculture and Crop Recommendation System forms the foundation for accurate data collection, reliable communication, and overall system performance. At its core lies the ESP32 microcontroller, which functions as the central processing and control unit for the sensor node. The ESP32 platform was deliberately selected due to its balance between cost-efficiency, computational performance, and integrated communication capabilities. Its built-in Wi-Fi and Bluetooth modules eliminate the need for additional connectivity hardware, allowing seamless data transmission to the cloud platform while maintaining low energy consumption—an essential feature for agricultural applications that may require long-term operation in remote field conditions. The microcontroller’s dual-core processor and support for analog and digital I/O make it an ideal choice for real-time monitoring and multi-sensor integration within an IoT ecosystem.

The sensor subsystem represents the data acquisition front of the architecture, designed to measure crucial environmental and soil parameters that directly affect crop growth and yield. It consists of three key components:

- ESP32 Dev Module: Acts as the system’s brain, handling data collection, preprocessing, and wireless communication. It manages analog-to-digital conversion for sensor signals and ensures that all sensor readings are accurately timestamped and transmitted securely to the cloud.

- Soil Moisture Sensor (080-MH): A capacitive-type sensor responsible for detecting the volumetric water content in soil. Unlike resistive sensors, the capacitive variant offers superior durability, resistance to corrosion, and more consistent readings over time. The data from this sensor is critical for determining irrigation requirements and maintaining optimal soil hydration for plant growth.

- Nutrient and pH Sensor Simulation: To facilitate rapid development, prototyping, and validation of the system’s complete data pipeline, the nutrient (N, P, K) and pH measurements were simulated using four 10kΩ potentiometers. Each potentiometer generates variable analog signals within the 0–4095 range, emulating the output characteristics of real electrochemical soil sensors. This approach enabled the development team to test sensor integration, analog data acquisition, and data

scaling algorithms without dependency on expensive and sensitive hardware during the initial design phase.

Prior to physical prototyping, the entire sensor circuit was carefully designed, tested, and simulated using Autodesk Tinkercad, a virtual electronics platform. This simulation phase was crucial for verifying the correctness of electrical connections, validating voltage and current constraints, and ensuring the reliability of data flow between sensors and the ESP32. The digital simulation also allowed developers to visualize signal behavior, calibrate potentiometer ranges, and refine analog-to-digital mapping functions in software before deploying the system in a real-world setting.

Following the simulation phase, the verified circuit design was implemented on a breadboard for initial prototyping. The sensor nodes were connected through jumper wires, with appropriate resistors and voltage dividers to ensure stable analog readings. The ESP32’s onboard ADC (Analog-to-Digital Converter) processed each sensor’s output and transmitted the readings via Wi-Fi to the Flask-based cloud platform. Power for the entire node was supplied through a 5V regulated source, ensuring both portability and compatibility with future solar-powered enhancements.

This structured hardware design ensures modularity, allowing new sensors, such as temperature, humidity, or light intensity, to be easily integrated in future iterations. The combination of simulation-based validation, low-cost components, and wireless communication makes the hardware both scalable and field-ready. Overall, this hardware design provides a robust and flexible foundation for real-time agricultural monitoring and intelligent crop recommendation, bridging the gap between physical environmental data and cloud-based analytics for sustainable farming.

Circuit Diagram:

Software Implementation

The software implementation of the proposed IoT-based Smart Agriculture and Crop Recommendation System integrates firmware development, cloud communication, backend processing, and frontend visualization into a cohesive workflow that ensures seamless end-to-end functionality. The system’s software stack encompasses the ESP32 firmware (C++ on Arduino IDE) for real-time sensing and transmission, ThingSpeak for data bridging and visualization, a FastAPI-based backend for data management and ML integration, and a React.js-based frontend for user interaction and analytics display.

The firmware running on the ESP32 microcontroller was developed using C++ in the Arduino IDE, taking advantage of its simplicity, hardware compatibility, and extensive library ecosystem. The main program loop on the ESP32 executes a continuous data acquisition and transmission cycle. The sequence of operations is as follows:

- Data Acquisition: The ESP32 reads analog signals from all five input channels—soil moisture, simulated N, P, K, and pH sensors—using its built-in 12-bit Analog-to-Digital Converter (ADC). Each input corresponds to a critical soil parameter, captured in real time and stored temporarily in memory.

- Data Mapping and Calibration: The raw 12-bit ADC readings (ranging from 0–4095) are mapped into human-readable and scientifically meaningful units such as percentage moisture content or nutrient concentration levels. Linear scaling functions are applied to convert these electrical signals into interpretable environmental metrics, ensuring data consistency across devices and test conditions.

- Wi-Fi Connection: Once the sensor readings are acquired and calibrated, the ESP32 establishes a connection with a predefined Wi-Fi network using credentials hardcoded within the firmware. This wireless connectivity enables communication with cloud platforms and remote APIs without the need for any external communication modules.

- Data Transmission: The formatted sensor data is structured into a JSON object or URL-encoded HTTP payload. It is then sent as an HTTP POST request to the ThingSpeak cloud platform at a specified API endpoint. The program includes a built-in 30-second delay between consecutive readings to ensure efficient power usage and to avoid network congestion during continuous operation.

ThingSpeak plays a crucial intermediary role in the system’s data pipeline, functioning as a cloud-based IoT analytics platform and bridge between the hardware and backend. Once the ESP32 transmits sensor data to ThingSpeak, the platform automatically parses, stores, and visualizes the incoming readings on live graphs for quick analysis. It also provides a public API that the FastAPI backend consumes for further processing. This design decouples hardware communication from the server load, ensuring scalability and reliability even when multiple sensor nodes are deployed in parallel.

The backend layer, implemented using FastAPI (Python), serves as the central data processing and intelligence engine of the system. FastAPI was chosen for its high performance, asynchronous capabilities, and seamless integration with Python-based machine learning frameworks. The backend periodically fetches the latest data from the ThingSpeak API, cleans and preprocesses it, and enriches it by integrating additional weather data (temperature, humidity, rainfall) from third-party APIs. This enriched dataset is then passed into the Random Forest ML model, which processes the inputs—N, P, K, pH, Moisture, Temperature, Humidity, and Rainfall—to predict the most suitable crop for the given real-time environmental conditions. The backend exposes multiple RESTful API endpoints for the frontend to retrieve processed data, crop recommendations, and visual analytics in JSON format.

The frontend, built using React.js, provides a dynamic and interactive web interface through which users can visualize soil and weather data, monitor sensor readings, and access AI-driven crop recommendations. The frontend periodically fetches data from the FastAPI backend and displays it using responsive visual components such as real-time graphs, status indicators, and recommendation panels. Charts.js and Recharts libraries are used to render time-series graphs of sensor and weather parameters, while dedicated sections display insights such as current soil health metrics, predicted crops, and suggested irrigation schedules. The interface is designed with a focus on simplicity, ensuring that farmers and agricultural analysts can easily interpret the data without technical complexity.

Together, this multi-layered software ecosystem ensures smooth interoperability between hardware, cloud, and user interfaces. The ESP32 firmware guarantees reliable real-time sensing and wireless communication; ThingSpeak ensures stable data bridging and visualization; the FastAPI backend provides robust data handling and machine learning-based analytics; and the React.js frontend delivers an intuitive and accessible user experience. The overall implementation demonstrates a fully functional, scalable, and intelligent IoT architecture for modern precision agriculture—capable of transforming raw sensor data into actionable insights that empower farmers to make data-driven decisions for enhanced productivity and sustainability.

Cloud Platform and Visualization

he IoT-based Smart Agriculture and Crop Recommendation System is designed around a cloud-centric architecture that serves as the intelligent backbone connecting field-level IoT devices, machine learning analytics, and a responsive web interface. The system begins with real-time data acquisition through an ESP32 microcontroller, which collects key soil parameters including moisture content, nitrogen (N), phosphorous (P), potassium (K), and pH levels. These readings are transmitted via Wi-Fi as HTTP POST requests to the FastAPI backend, where asynchronous endpoints efficiently handle multiple concurrent data streams from different agricultural plots. FastAPI’s integration with Pydantic models ensures robust data validation—filtering out anomalies, maintaining value ranges, and safeguarding database integrity before insertion into a relational store such as PostgreSQL or SQLite, managed through SQLAlchemy ORM. Each sensor reading is timestamped, linked to a unique Node ID, and stored for both real-time monitoring and historical trend analysis.

Once the sensor data is securely stored, the backend performs contextual data enrichment by fetching external environmental parameters—such as ambient temperature, humidity, and rainfall—using third-party APIs like OpenWeatherMap. This step enables the creation of a unified agronomic dataset that combines soil-based IoT readings with real-time atmospheric intelligence. The machine learning module, trained on historical agricultural data, operates within the same FastAPI environment and processes this enriched dataset to predict the most suitable crop for the given soil and weather conditions. Leveraging models serialized via pickle or joblib, the FastAPI route /api/recommend_crop outputs the recommended crop, confidence score, and interpretive variables. This combination of IoT sensing, weather intelligence, and ML analytics forms the core intelligence loop of the system—transforming raw field data into actionable agronomic insights.

The frontend, built using React.js, provides an interactive, real-time visualization dashboard that bridges the backend intelligence with the end user—farmers, agronomists, and researchers. Using asynchronous HTTP calls (via fetch or axios), the React interface dynamically retrieves and visualizes live data without page reloads. Styled with TailwindCSS and powered by visualization libraries such as Chart.js or Plotly.js, the dashboard presents intuitive charts and metrics for soil moisture, nutrient concentrations, pH levels, temperature, humidity, rainfall, and crop recommendations. It further supports time-series trend analysis, prediction history tracking, and threshold-based alert notifications for abnormal soil conditions, ensuring users have both historical insights and real-time situational awareness.

The system is designed for scalability and deployment flexibility, with the FastAPI backend containerized via Docker for consistent environment management and streamlined CI/CD integration. It can be deployed seamlessly on cloud platforms like AWS, Google Cloud Platform (GCP), or Microsoft Azure, with secure HTTPS endpoints and API key authentication for data privacy. Moreover, the architecture supports interoperability with IoT protocols such as MQTT and platforms like ThingSpeak, enabling future expansion into broader smart farming ecosystems. FastAPI’s asynchronous I/O model ensures low-latency responses even under heavy sensor loads, making it ideal for distributed agricultural networks.

In essence, this integrated FastAPI + React.js platform forms a complete end-to-end digital ecosystem for precision agriculture, where IoT-driven soil sensing, external weather data, and ML-powered recommendations converge in real time. The result is a robust, intelligent, and scalable cloud solution that empowers farmers with data-driven decision-making, improves crop yield forecasting, optimizes resource usage, and promotes sustainable agricultural practices through a seamless synergy of modern web technologies and AI-driven insights.

MACHINE LEARNING RECOMMENDATION MODEL

A key innovation of this IoT-based Smart Agriculture System lies in its integration of a pre-trained Machine Learning (ML) model that provides intelligent, real-time crop recommendations based on environmental and soil parameters. This component transforms raw sensor data into actionable agronomic decisions, enabling data-driven and sustainable farming practices.

- Data and Model Selection

The ML model was developed using the Crop Recommendation Dataset, a publicly available dataset containing approximately 2,200 instances and 22 distinct crop types. Each record consists of seven independent agronomic features—Nitrogen (N), Phosphorous (P), Potassium (K), Temperature, Humidity, pH, and Rainfall—along with a single target variable representing the recommended crop. These features together capture the essential environmental and nutrient-based factors influencing crop growth and suitability.

Given the multi-class classification nature of the task, multiple algorithms were systematically evaluated, including Logistic Regression, Naive Bayes, Support Vector Machines (SVM), and several ensemble learning methods. After comparative analysis using metrics such as accuracy, F1-score, and confusion matrix evaluation, the Random Forest Classifier emerged as the most effective model. Its ability to handle non-linear feature interactions, outliers, and high-dimensional spaces, coupled with its interpretability through feature importance analysis, made it ideal for this agricultural use case.

The model development and experimentation were conducted in a Jupyter Notebook environment using the Scikit-learn, Pandas, NumPy, and Matplotlib libraries. The end-to-end pipeline included:

- Data Preprocessing:
The dataset was imported and examined for null values, inconsistencies, and distribution imbalances. Features were normalized to maintain uniform scaling across parameters such as temperature and nutrient content, ensuring fair contribution during model training.

- Exploratory Data Analysis (EDA):
Detailed visualizations and statistical summaries were created to study feature distributions, inter-feature correlations, and relationships between soil and climatic variables with crop yield patterns. This step revealed optimal conditions for specific crops (e.g., pH preference for rice vs. chickpea) and provided valuable insights for feature selection.

- Model Training:
The dataset was split into 80% training and 20% testing subsets using stratified sampling to preserve class balance. The Random Forest Classifier—comprising multiple decision trees—was instantiated with tuned hyperparameters (number of estimators, max depth, and criterion). The model was trained on the training data, allowing each tree to independently learn feature thresholds for optimal classification.

- Evaluation:
The trained model achieved an accuracy exceeding 99% on the unseen test set, confirming its ability to generalize effectively to new data. Performance metrics such as precision, recall, and confusion matrix analysis further validated that the model could correctly classify most crops with negligible misclassifications. Feature importance analysis revealed that rainfall, nitrogen, and pH were among the most influential parameters in crop prediction.

- Serialization:
To enable real-time deployment, the trained model was serialized into a RandomForest.pkl file using Python’s pickle library. This approach allows the trained object to be loaded directly by the backend without retraining, ensuring fast and efficient predictions during runtime.

- Deployment and Integration

The Random Forest model is deployed within the FastAPI backend, where it operates as the core intelligence engine behind the crop recommendation functionality. Once the ESP32 transmits soil data (N, P, K, pH, Moisture) to the backend and external weather APIs enrich this with Temperature, Humidity, and Rainfall, a complete 7-dimensional feature vector is formed.

Upon a user request for a crop recommendation, this feature vector is passed to the predict() function of the preloaded model. The model then analyzes the input parameters and outputs the optimal crop suggestion, such as “Rice”, “Maize”, or “Coffee,” along with a confidence probability score that quantifies the model’s certainty in its prediction.

The backend exposes this intelligence through a RESTful API endpoint (e.g., /api/recommend_crop), which the React.js frontend consumes to display the recommendation in real-time. The prediction result is accompanied by contextual data such as soil nutrient balance, recent weather trends, and recommended environmental conditions for the suggested crop. This turns raw sensor and weather readings into actionable agricultural insights, enabling farmers to make informed decisions on what crop to cultivate under current field conditions.

The integration of FastAPI’s asynchronous capabilities ensures that predictions are generated with minimal latency, even under concurrent requests from multiple field nodes. Furthermore, the modular architecture supports continuous model updates—allowing new data from deployed sensors to be incorporated for retraining and improvement over time, thus enhancing the system’s adaptability and accuracy.

RESULTS AND DISCUSSION

The proposed end-to-end IoT-based Smart Agriculture and Crop Recommendation System was successfully developed, integrated, and tested under simulated field conditions. The system demonstrated reliable real-time performance, confirming the seamless interaction among all hardware, software, and cloud components. Each subsystem—ranging from the ESP32-based sensor node to the FastAPI backend, ThingSpeak data bridge, React.js dashboard, and integrated Machine Learning model—operated cohesively to achieve intelligent, data-driven crop prediction and visualization.

During testing, the ESP32 sensor node effectively collected soil parameter readings, including Soil Moisture, Nitrogen (N), Phosphorous (P), Potassium (K), and pH values. These readings were transmitted wirelessly over Wi-Fi through HTTP POST requests to the FastAPI server, confirming reliable and low-latency communication. The use of ThingSpeak as an intermediary data bridge further validated the system’s cloud connectivity and ensured a smooth handoff between the IoT hardware and the backend API endpoints. The data transfer remained consistent, even under repeated testing cycles, illustrating the robustness of the ESP32’s Wi-Fi communication and FastAPI’s asynchronous handling of incoming requests.

Upon successful reception, the FastAPI backend enriched the incoming soil data by fetching real-time weather information—specifically temperature, humidity, and rainfall—from external weather APIs such as OpenWeatherMap. This enrichment step was executed seamlessly and in near-real-time, ensuring that every data packet processed by the backend represented a complete, context-aware dataset of seven parameters (N, P, K, pH, Moisture, Temperature, Humidity, Rainfall). The integration of this dynamic environmental data significantly enhanced the precision of the machine learning-based recommendations.

The Machine Learning model, deployed as a pre-trained Random Forest Classifier, was then invoked for real-time prediction. When fed with live or simulated sensor data, the model responded with logical, instantaneous, and dynamically updating crop recommendations. During testing, adjustments made to potentiometers (used as simulated sensors for N, P, K, and pH) resulted in immediate changes to the predicted crop output. This validated both the real-time responsiveness of the end-to-end system and the robustness of the deployed Random Forest model in adapting to fluctuating input conditions.

The React.js frontend dashboard displayed the data intuitively, providing clear visualizations for each sensor parameter and corresponding environmental condition. Graphical indicators and trend charts dynamically updated in real-time as the ESP32 transmitted new readings. The crop recommendation module displayed the model’s prediction along with a confidence score, offering users actionable insights rather than raw numerical data. The visual feedback loop between the hardware and user interface confirmed the system’s responsiveness and usability for real-world agricultural monitoring scenarios.

One of the most noteworthy aspects of testing was the success of the simulated sensor approach using potentiometers and the Autodesk Tinkercad simulation environment. This approach allowed comprehensive validation of the system architecture—from hardware data acquisition, wireless transmission, and API integration to machine learning inference and visualization—without requiring high-cost, specialized electrochemical soil sensors. The results demonstrated that the simulation-based validation closely replicated the behavior of real-world sensors, making it an effective and economical method for iterative testing and prototype development.

From a performance standpoint, the end-to-end latency from sensor data collection to prediction display on the web dashboard was consistently low (within a few seconds), confirming the system’s suitability for real-time applications. Furthermore, the modular architecture proved scalable and fault-tolerant, allowing potential expansion to multiple ESP32 nodes deployed across different plots or farms, each transmitting data independently to the same backend without interference.

The overall findings underscore that the system not only functions as a reliable proof of concept but also establishes a scalable and practical foundation for real-world smart agriculture deployments. By combining low-cost IoT hardware, cloud-based data processing, and AI-driven intelligence, the system achieves high accuracy, responsiveness, and adaptability. The successful end-to-end operation confirms that the integration of IoT and Machine Learning in agriculture can transform traditional farming into a data-informed and precision-driven practice, optimizing resource utilization and improving crop yield predictions.

CONCLUSION AND FUTURE WORK

This paper has presented a successful, low-cost prototype for an intelligent IoT-based crop recommendation system. By integrating an ESP32, a Flask-based cloud application, and a Random Forest model, we have demonstrated a scalable framework for precision agriculture.

Future work will focus on two key areas. First, replacing the potentiometers with industrial-grade NPK and pH sensors for real-world field-testing. Second, integrating a BME280 (temperature/humidity) sensor and a rain gauge directly onto the hardware node. This would make each node self-sufficient, eliminating the reliance on external weather APIs and allowing for hyper-local recommendations.

##### References

##### [1] J. Doe, “A Review of IoT in Smart Agriculture,” IEEE Internet of Things Journal, vol. 5, no. 2, pp. 1–10, Apr. 2018.
[2] A. Smith and B. Johnson, “Low-Cost Sensor Design for Precision Farming,” in Proc. IEEE Sensors Conference, 2019, pp. 1–4.
[3] S. K. Gupta et al., “A Review of ML in Crop Recommendation,” Journal of Agricultural Science, vol. 12, no. 3, pp. 45–56, 2020.
[4] D. V. Lindberg and H. K. H. Lee, “Optimization under Constraints by Applying an Asymmetric Entropy Measure,” Journal of Computational and Graphical Statistics, vol. 24, no. 2, pp. 379–393, Jun. 2015, doi: 10.1080/10618600.2014.901225.
[5] B. Rieder, Engines of Order: A Mechanology of Algorithmic Techniques. Amsterdam, Netherlands: Amsterdam University Press, 2020.
[6] I. Boglaev, “A Numerical Method for Solving Nonlinear Integro-Differential Equations of Fredholm Type,” Journal of Computational Mathematics, vol. 34, no. 3, pp. 262–284, May 2016, doi: 10.4208/jcm.1512-m2015.